# ELEC 301 – Mini Project 1

# Part I

The poles are each a decade apart from each other, so we can work under the assumption that each pole is equal to the inverse time constant of a single capacitor. From the condition that C1 > C2 > C3 we see that C1 is responsible for the corner frequency ω = 10, C2 is responsible for the corner frequency ω = 100, and C3 is responsible for the corner frequency ω = 1000.

First, we find the capacitance of C1. At ω = 10, only C1 is operating, so C2 and C3 are seen as open circuits.

RC1=2kΩ

τc1=2000*C1=1ωC1=110

C1=50 μF

Next, we find the capacitance of C2. At ω = 100, C1 is now conducting and is seen as a short circuit while C3 is seen as an open circuit.

RC2=1kΩ+1kΩ || 1kΩ =  1500Ω

τC2=1500*C2=1ωC2=1100

C2=6.67 μF

Finally, we find the capacitance of C3. At ω = 1000, C1 and C2 are now conducting and are seen as short circuits.

RC3=1kΩ+1kΩ ||1kΩ ||1kΩ=1333.3Ω

τC3=1333.3*C3=1ωC3=11000

C3=0.75 μF

In summary, from the Method of Open Circuit and Short Circuit Time Constants, we have found:

C1=50.0 μF

C2=6.67 μF

C3=0.75 μF

Now, we simulate the circuit along with a transfer function equivalent to the one in the problem specification:

where Ts=0.25*ss+10*ss+100*ss+1000

Bode plots were generated for both of these simulations and compared. Below are the magnitude and phase bode plots. The simulation of the SXFER block is plotted in black, while the simulation of the circuit is plotted in red. The output plots are virtually indistinguishable, confirming the validity of our approximation.

# Part II

We start by using Circuit Maker to simulate the circuit as specified.

A magnitude and phase Bode plot were generated from this simulation.

We use the magnitude bode plot to locate poles by extending the regions of constant slope and finding their intersection points.

From this graphical analysis, poles are estimated to be located at:

ωLP1≈7 Hz

ωLP2≈80Hz

ωHP1≈2.5 MHz

ωHP2≈48 MHz

Now, we extend this graphical analysis to find low frequency poles and 3 dB points for C3 values of 1μF, 2μF, 5μF, 10μF.

The Bode plot below is the same as above – the magnitude Bode plot for the case where C3 = 1μF. We use this plot to estimate the 3 dB points by placing one horizontal cursor along the midband gain and the other horizontal cursor 3 dB below. The 3 dB points occur where this second horizontal line crosses the transfer function. We use the vertical cursors to find the frequency values of these positions.

ω3dBL1μF=79.88 Hz

ω3dBH=2.989 MHz

The same parameters are likewise extracted from C3 values of 2μF, 5μF, 10μF. First 2μF:

ωL12μF≈5.5 Hz

ωL22μF≈48 Hz

ω3dBL2μF=44.29 Hz

Next, 5μF:

ωL15μF≈4.3 Hz

ωL25μF≈23 Hz

ω3dBL5μF=22.58 Hz

And finally, 10μF:

ωL110μF≈3.9 Hz

ωL210μF≈18 Hz

ω3dBL10μF=15.67 Hz

For direct comparison, here are the magnitude and phase Bode plots for all capacitor configurations overlaid. C3 = 1μF is represented by purple, C3 = 2μF is represented by cyan, C3 = 5μF red and C3 = 10μF green.

Now, the 3 dB points of the circuit are estimated using the method of Open-Circuit and Short-Circuit Time Constants.

First, we analyze ω3dBH. At high frequencies, the microfarad scale capacitors will have long since been conducting, and are treated as short circuits. We make this adjustment to the circuit, calculate the open-circuit time constants of the high frequency capacitors, and use these to estimate ω3dBH.

First we find the open-circuit time constant for C2.

RC2=(R2|R1||R3+R4=(1000|50||1000+1000=46.51 Ω

τC2OC=C2RC2=100pF46.51Ω=4.65 ns

Then, we find the open-circuit time constant for C4.

RC4=R4||(R3+R2|R1=1000||(1000+1000|50=511.63Ω

τC4OC=C4RC4=100pF511.63Ω=51.16 ns

We use these values to solve for ω3dBH.

ω3dBH≈1i=1nτiOCso ω3dBH≈151.16 ns+4.65 ns

ω3dbH≈17.92Mrads=2.85 MHz

Since we do not vary the values of C2 or C4, we can be sure that this value is constant for every configuration of Part II.

We then look at ω3dBL. At the low breakpoint frequencies, the two 100 pF capacitors will appear as open circuits. With this in mind, we calculate the short-circuit time constants of the low frequency capacitors and use these to estimate ω3dBL. Since we will be varying the capacitance of C3, we leave it as a variable.

First, we find the short-circuit time constant for C1.

RC1=R1+R2R3+R4=50+10001000+1000=716.67Ω

τC1SC=C1RC1=20μF716.67Ω=0.0143 s

Next, we find the short-circuit time constant for C3.

RC3=R4+R3+(R2|R1=1000+1000+(1000|50=2047.62Ω

τC3SC=C3RC3=C32047.62

We use these values to solve for ω3dBL.

ω3dBL≈ i=1n1τCiSC  so ω3dBL≈10.0143s+12047.62*C3

Substituting in C3 = 1μF, 2μF, 5μF, 10μF:

ω3dBL1μF≈556.73rads=88.77 Hz

ω3dBL2μF≈313.83rads=49.95 Hz

ω3dBL5μF≈167.59rads=26.67 Hz

ω3dBL10μF≈118.76rads=18.90 Hz

The results from Part II are summarized in the following table. % Error is calculated as

%Error=100*ωgraphical- ωcalculatedωgraphical

| C3 | ω3dBL (Hz) (calculated) | ω3dBL (Hz) (Graphically) | % Error | ω3dBH (Hz) (Graphically) | ωL1 (Hz) (Graphically) | ωL2 (Hz) (Graphically) |
| --- | --- | --- | --- | --- | --- | --- |
| 1μF | 88.77 | 79.88 | 11.13% | 2.99 M | 7 | 80 |
| 2μF | 49.95 | 44.29 | 12.78% | 2.99 M | 5.5 | 48 |
| 5μF | 26.67 | 22.58 | 18.11% | 2.99 M | 4.3 | 23 |
| 10μF | 18.90 | 15.67 | 20.61% | 2.99 M | 3.9 | 18 |

An observation: as the locations of the low frequency poles become closer together, our approximation of Open-Circuit and Short-Circuit Time Constants leads to a greater percent error. This is consistent with our assumption that this provides a reasonably accurate value provided that the next nearest poles are at least two octaves away. In fact, we can see that for 10μF:

ωL2ωL1=183.9=4.61

And for 5μF:

ωL2ωL1=234.3=5.35

Which are ratios both less than two octaves. Luckily, this is a fairly simple circuit, so we are able to approach each of the poles individually.

The calculation for ω3dBL is done over, using a more specific method than the sum of inversed short-circuit time constants.

From earlier, we saw that

τC1SC=0.0143 s

and

τC3SC=C3RC3=C32047.62

τC3SC < τC1SC for C3 = 1, 3 and 5 μF, so when finding the low frequency poles at this range, we say that C1 conducts before C3.

i) This means that we need to re-do our calculation for τC1 when C3 = 1, 3 and 5 μF, treating C3 as an open circuit.

RC1=R1+R2=1050Ω

τC1=C1RC1=20μF1050Ω=0.021 s for C3=1,3,5 μF

When C3 = 10 μF, τC3SC > τC1SC, so our earlier short circuit approximation for τC1 = 0.0143s still applies.

τC1=0.0143s for C3=10 μF

ii) This also means that we need to re-do our calculation for τC3 when C3 = 10 μF, treating C1 as an open circuit.

RC3=R4+R3+R2=1000+1000+1000=3000Ω

τC3=C3RC3=10μF3000=0.03s for C3=10μF

When C3 = 1, 3 and 5 μF, our earlier short circuit approximation for τC3 = C32047.62 still applies.

τC3=0.00205 for C3=1μF

τC3=0.00410  for C3=2μF

τC3=0.0102 for C3=5μF

iii) Finally, we are able to calculate ωL3dB

ωL3dB≈1τC3+1τc1

C3=1μF: ωL3dB≈ 535.42rads=85.21 Hz

C3=2μF: ωL3dB≈ 291.52rads=46.39 Hz

C3=5μF: ωL3dB≈ 145.65rads=23.18 Hz

C3=10μF: ωL3dB≈ 103.23rads=16.43 Hz

Plugging these values in to the table from before, we see substantially reduced error. This proves that more circuit specific methods are necessary to estimate 3dB points than OC/SC Time-Constants when consecutive poles are near another.

| C3 | ω3dBL (Hz) (recalculated) | ω3dBL (Hz) (Graphically) | % Error |
| --- | --- | --- | --- |
| 1μF | 85.21 | 79.88 | 6.67% |
| 2μF | 46.39 | 44.29 | 4.67% |
| 5μF | 23.18 | 22.58 | 2.66% |
| 10μF | 16.43 | 15.67 | 4.85% |

# Part III

We first find the poles of the low frequency capacitors. At these low frequencies, the conductance of the pF scale capacitors will be negligible, so we treat them as open circuits for our low frequency analysis.

The time constants of these capacitors are independent of the state of the other capacitor. Therefore, we can say that the two low frequency poles are equal to the inverse time constants of each capacitor.

First, we find the low frequency pole derived from time constant of C1 (the capacitor on the left).

RC1=1000+50=1050Ω

τC1=RC1C1=1050Ω1μF=0.00105s

ωLPC1=1τC1=952.38rads=151.58 Hz

Next, we find the low frequency pole derived from the time constant of C4 (the capacitor on the right). When finding the constant, we ignore the current source, thus treating it as an open circuit.

RC4=4000+4000=8000Ω

τC4=RC4C4=8000Ω1μF=0.00800 s

ωLPC4=1τC4=125.00rads=19.89 Hz

V1

We then calculate the mid-band gain. At these frequencies, the μF scale capacitors will be conducting, while the impedance on the pF scale capacitors will still not have broken, and thus they will still appear as open circuits.

The midband gain will be equal to Vo/Vi in these conditions.

Vo=-0.1*V1*(4000|4000=-200V1

Voltage Divider:V1=Vi1000+501000=0.95Vi

Vo= -2000.95Vi= -190.5Vi

Am=VoVi= -190.5VV

Finally, we find the poles of the high frequency capacitors. At high frequencies, the μF scale capacitors will be conducting and can be treated as short circuits.

The 2pF capacitor is now active, so we can no longer treat the system as two independent circuits. In order to simplify analysis, Miller’s theorem is applied.

First, we find the Thevenin equivalent of the right hand side so we can establish a V2 = kV1 relationship.

Zmiller

Integrating this into our full circuit:

This is a circuit that we can apply Miller’s Theorem to. V2 = -200V1 so our Miller Gain k = -200. Follows is the calculation for the new impedances to be added in parallel to each side.

Z=2000+1s*2pF

Z1=Z1-k=Z*1201=>9.95Ω and 0.402nF

Z2=Zkk-1=Z*200201=>1990Ω and 2.01pF

Following these calculations, we obtain the equivalent circuit:

As an approximation, we ignore the 9.95Ω resistor, as it is negligible compared to the internal resistance. This allows us to simplify the capacitors in parallel on the left hand side to an equivalent single capacitor. In making this approximation, we realize that we will be neglecting a zero that occurs close to our highest frequency pole. Since we will not be operating at frequencies in the ~10GHz range, this is acceptable.

We are now able to find time constants for both of the high frequency capacitors. The time constants of these capacitors are independent of the state of the other capacitor. Therefore, we can say that the two high frequency poles are equal to the inverse time constants of each capacitor.

First, we find the high frequency pole determined by the time constant of the .412nF capacitor:

RCH1=1000||50=47.62 Ω

τCH1=RCH1CH1=47.62Ω0.412nF=1.96*10-8s

ωH1=1τCH1=50.97Mrads=8.11 MHz

Next, we find the high frequency pole determined by the time constant of the 2.01pF capacitor:

RCH2=1990 Ω

τCH2=RCH2CH2=1990Ω2.01pF=4.00*10-9 s

ωH2=1τCH2=250.01Mrads=39.79 MHz

Now that we have calculated the poles and the midband gain, we can assemble an approximation of the transfer function. Due to the band-pass behavior of this circuit, we know that there are two zeros located at ω = 0.

VosVis= -190.5*ss+125*ss+952.38*50.97*106s+50.97*106*250.01*106s+250.01*106

From the estimated poles, approximations of the 3dB points are calculated:

ω3dBL≈ ωL12+ ωL22= 952.382+1252=960.55rads=152.88 Hz

ω3dBH≈1τ3dbHwhere τ3dBH=τH12+τH22= 1.96*10-82+4.00*10-92=1.95*10-8

ω3dBH≈49.94Mrads=8.15 MHz

Now we compare our values found through these approximate analysis techniques to a simulation of the circuit of interest:

The phase Bode plot was generated:

The magnitude bode plot was plotted, and was used to graphically identify the 3dB points as well as all poles and zeroes. The same methods were used as in Part II.

ωPL1≈21 Hz

ωPL2≈180 Hz

ωPH1≈7.1 MHz

ωPH2≈ 1.5 GHz

ωZH≈5.8 GHz

ω3dBL≈149.1 Hz

ω3dBH≈6.506 MHz

The following table compares the 3-dB points and poles from simulation and calculation.

| C3 | Calculated | Simulated | % Error |
| --- | --- | --- | --- |
| ω3dBL (Hz) | 152.88 | 149.1 | 2.53% |
| ω3dBH (Hz) | 8.15 M | 6.506 M | 20.21% |
| ωPL1 (Hz) | 19.89 | 21 | 5.29% |
| ωPL2 (Hz) | 151.58 | 180 | 15.79% |
| ωPH1 (Hz) | 8.11 M | 7.1 M | 14.23% |
| ωPH2 (Hz) | 39.79 M | 1.5 G | 97.30% |
| ωZH (Hz) | - | 5.8 G | - |

As expected, we see the high frequency zero that we abstracted away in our earlier approximation only in graphical analysis. This approximation also shows an effect on our calculation of ωPH2, which is acceptable since we never expect to be operating at these frequencies.